Water Quality Analysis Report

# 1. Dataset Overview

The cleaned dataset contains 3,276 water samples across 9 chemical parameters. After cleaning, all missing values in pH (491), Sulfate (781), and Trihalomethanes (162) were filled using median imputation, ensuring no data loss.

| Parameter | Min | Max | Mean | Std Dev |
| --- | --- | --- | --- | --- |
| pH | 1.93 | 13.21 | 7.14 | 1.44 |
| Hardness (mg/L) | 74.42 | 314.72 | 196.71 | 32.87 |
| Solids (mg/L) | — | 49,597 | 21,831 | 8,887 |

# 2. Class Distribution

Out of 3,276 samples:

- Not Potable (Unsafe): 1,966 samples → 60.0%

- Potable (Safe): 1,310 samples → 40.0%

Inference: The dataset is imbalanced — 3 out of every 5 water samples are unsafe. This means any ML model trained on this data without handling imbalance will naturally be biased toward predicting "unsafe." This is why Logistic Regression accuracy stays around 60–62% — it's essentially learning the majority class.

# 3. WHO Standard Violations

| Parameter | WHO Limit | Samples Violating | % Violating |
| --- | --- | --- | --- |
| Solids | < 500 mg/L | 3,256 | 99.4% |
| Sulfate | < 250 mg/L | 3,225 | 98.4% |
| Chloramines | < 4 mg/L | 3,204 | 97.8% |
| Conductivity | < 400 µS/cm | 2,062 | 62.9% |
| pH | 6.5 – 8.5 | 1,472 | 44.9% |
| Trihalomethanes | < 80 µg/L | 635 | 19.4% |
| Turbidity | < 5 NTU | 308 | 9.4% |

Inference: Nearly the entire dataset violates WHO limits for Dissolved Solids, Sulfate, and Chloramines. This strongly suggests the data represents heavily contaminated or industrial water sources, not regular municipal water. Turbidity is the only parameter where most samples (90.6%) are within safe range.

# 4. pH Analysis

- Overall mean pH: 7.14 — technically within neutral range

- But 44.9% of all samples fall outside the WHO safe range of 6.5–8.5

- pH ranged from 1.93 (highly acidic) to 13.21 (highly alkaline) — extreme values indicating severe contamination in edge cases

pH Group Breakdown:

| pH Group | Total Samples | Safe | Unsafe | % Safe |
| --- | --- | --- | --- | --- |
| Acidic (< 6.5) | 954 | 393 | 561 | 41.2% |
| Neutral (6.5–8.5) | 1,804 | 695 | 1,109 | 38.5% |
| Alkaline (> 8.5) | 518 | 222 | 296 | 42.9% |

Inference: Surprisingly, pH group has almost no impact on potability in this dataset. Safe water percentages are nearly identical across Acidic (41.2%), Neutral (38.5%), and Alkaline (42.9%) groups. This tells us pH alone is not a reliable predictor of whether water is safe. . Water treatment cannot rely on pH correction alone.

# 5. Chemical Comparison

| Parameter | Safe Water Mean | Unsafe Water Mean | Difference |
| --- | --- | --- | --- |
| pH | 7.146 | 7.132 | +0.014 |
| Hardness | 196.71 | 196.71 | ~0 |
| Solids | 21,689 | 21,925 | −236 |
| Chloramines | 7.115 | 7.106 | +0.009 |
| Turbidity | 3.954 | 3.972 | −0.018 |

Inference: The means of all 9 parameters are almost identical between safe and unsafe water. There is no single chemical parameter that clearly separates potable from non-potable water in this dataset. This is statistically confirmed by correlation analysis — the highest correlation any parameter has with Potability is just −0.013 (Solids). This is a critical finding — it explains why simple ML models struggle on this dataset and why real-world water testing requires multi-parameter combination analysis, not individual thresholds.

# 6. Correlation Analysis

All 9 parameters have near-zero correlation with Potability:

| Parameter | Correlation with Potability |
| --- | --- |
| Solids | −0.013 |
| Turbidity | −0.012 |
| Organic Carbon | +0.010 |
| Conductivity | +0.007 |
| pH | +0.005 |
| Chloramines | +0.003 |
| Trihalomethanes | −0.002 |
| Sulfate | ~0.000 |
| Hardness | ~0.000 |

Inference: No linear relationship exists between any individual chemical parameter and water potability. This is a non-linear classification problem — meaning simple Logistic Regression will underperform, and this is expected and acceptable. A tree-based model like Random Forest would perform better.

# 7. Outlier Analysis

All 9 parameters showed very low outlier counts (< 0.5% each), meaning the cleaned dataset is statistically well-behaved. However one important anomaly:

Solids column contains a negative value (−13,534 mg/L) — which is physically impossible since dissolved solids cannot be negative. This is a data quality issue in the original Kaggle dataset itself.

# 8. Key Conclusions

1. Water scarcity of safe water is severe — Only 40% of sampled water is potable, meaning 60% fails safety standards. This reflects a real global water quality crisis.

2. No single parameter predicts potability — All correlations are below 0.02, meaning water safety is determined by the combined effect of multiple parameters, not any one chemical. This aligns with real ChE water treatment principles.

3. Dissolved Solids is the most widespread violation — 99.4% of samples exceed WHO limits for TDS. This indicates the dataset likely represents groundwater or industrial sources with high mineral content.

4. pH is not a reliable safety indicator — Safe and unsafe water show nearly identical pH distributions across Acidic, Neutral, and Alkaline groups. This challenges the common assumption that neutral pH = safe water.

5. Model limitation is dataset-driven — The ~62% Logistic Regression accuracy is not a coding failure — it reflects the genuine complexity of the problem. Even advanced models on this dataset typically cap at 67–70%, which you can cite from Kaggle discussions.

# 9. Limitations

- Dataset source and collection methodology are not disclosed on Kaggle — geographic origin unknown

- Class imbalance (60:40) affects model training

- Missing values in pH, Sulfate, Trihalomethanes were imputed with median — introduces slight bias

- One negative Solids value detected — likely a data entry error in source data

- Logistic Regression assumes linear separability — not valid here, hence limited accuracy

Prepared By - Krunal A Deshbhratar